# NBAD Capacity Planning and Sizing Workbook

OPTCL Next Generation SOC & NOC: Network Behaviour Anomaly Detection

Version 0.1 | Companion to the NBAD Project Blueprint | Draft for engineering review

Status: Worked examples and formulas only. Every figure is a scenario calculation. Do not use any number here as a procurement size until OPTCL’s real traffic has been measured and the benchmark definitions are agreed.

## 1. Purpose

The tender gives a few hard numbers (512 Mbps at the DC, 5 million records in 180 seconds, 7 days of raw packets, 15 days of normalized data, 30% spare capacity). This workbook shows how each one turns into storage, packet-rate and throughput requirements, and what inputs we still need before a size can be committed.

## 2. Raw packet storage

### 2.1 Formula

Daily raw storage = average captured bit rate ÷ 8 × 86,400 seconds.

Total raw storage = daily raw storage × retention days × (1 + planning margin) × replication factor, plus filesystem and index overhead.

### 2.2 Worked example at 512 Mbps, sustained, no filtering

| Step | Value |
| --- | --- |
| Bytes per second | 512,000,000 ÷ 8 = 64,000,000 |
| Per day | 5.53 TB (decimal) |
| Seven days | 38.7 TB |
| With a 30% planning margin | 50.3 TB |
| With a replication factor of 2 on top | about 100.6 TB, before filesystem overhead |

### 2.3 Effect of average utilization

The tender says at least 512 Mbps. If that is a peak and the average is lower, the retained volume drops proportionally. We need the real average to decide.

| Average rate as a share of 512 Mbps | Per day | Seven days | With 30% margin |
| --- | --- | --- | --- |
| 25% | 1.38 TB | 9.7 TB | 12.6 TB |
| 50% | 2.76 TB | 19.4 TB | 25.2 TB |
| 100% | 5.53 TB | 38.7 TB | 50.3 TB |

### 2.4 If the real rate is higher than the tender minimum

| Sustained rate | Per day | Seven days | With 30% margin |
| --- | --- | --- | --- |
| 512 Mbps | 5.53 TB | 38.7 TB | 50.3 TB |
| 1 Gbps | 10.8 TB | 75.6 TB | 98.3 TB |
| 2 Gbps | 21.6 TB | 151.2 TB | 196.6 TB |
| 10 Gbps | 108 TB | 756 TB | 982.8 TB |

The tender also says the platform must keep at least 30% spare capacity at peak. Whether the planning margin and the spare-capacity requirement are one and the same, or stack on top of each other, must be confirmed in the tender text. Applying both is the conservative reading.

### 2.5 Other levers

- Capture filters. Excluding approved bulk traffic (for example encrypted backup streams) reduces volume, but only with OPTCL’s written approval.

- Truncation. Storing only the first N bytes of each packet cuts volume sharply but loses payload evidence. Check what forensic use requires.

- Compression. Packet files compress modestly, and compression costs CPU during capture. Test before counting on it.

## 3. Packet rate and sensor sizing

Sensors are limited by packets per second as much as by bits per second.

| Average packet size | Packets per second at 512 Mbps (approximate) |
| --- | --- |
| 1,500 bytes on the wire | about 41,600 |
| 1,000 bytes | about 64,000 |
| 600 bytes | about 106,700 |
| Minimum-size frames (84 bytes on the wire) | about 761,900 |

Real traffic sits somewhere between these. The packet-size distribution of OPTCL’s Internet edge decides how hard the sensors must work. Test at the measured mix and at a harsher, small-packet mix to find the headroom.

### Sensor checklist

- Required capture rate in Mbps and packets per second at peak, plus 30% headroom.

- NIC model, driver and capture method tested on the exact kernel version.

- CPU cores and memory per sensor from benchmark, not from vendor brochures.

- Disk write rate for local PCAP buffering.

- Capture-loss counters, with a target loss rate agreed in writing.

## 4. Flow and event storage

The flow rate is the unknown here. This example uses placeholder inputs.

| Input | Example value |
| --- | --- |
| Flow records per second (average) | 5,000 |
| Average serialized record size | 200 bytes |
| Retention | 15 days |

| Result | Value |
| --- | --- |
| Raw serialized data per second | 1,000,000 bytes |
| Per day | 86.4 GB |
| Fifteen days | about 1.3 TB |
| With a replication factor of 2 | about 2.6 TB |

Search indexes add overhead and compression subtracts from it. Both depend on the store and the schema, so measure them in the benchmark and replace this example with the result. If OPTCL’s flow rate is ten times higher, every number here is ten times higher.

## 5. Why “normalized packet data” must be defined

The 15-day retention requirement for normalized packet data could mean very different things. Example at an average packet size of 600 bytes and 512 Mbps (about 106,667 packets per second):

| Interpretation | Rough record rate | Rough size per day | Fifteen days |
| --- | --- | --- | --- |
| One metadata row per packet, about 100 bytes each | 106,667 rows per second | about 0.92 TB | about 13.8 TB |
| One record per session or flow | Likely a few thousand per second | Tens to low hundreds of GB | Low single-digit TB |
| Normalized copy of the full packet payload | Same as raw | about 5.53 TB | about 83 TB |

The spread is more than an order of magnitude. Ask OPTCL for the exact definition in writing before sizing.

## 6. The 5 million records in 180 seconds target

5,000,000 ÷ 180 = about 27,778 records per second.

What that means depends on what “retrieve” includes:

| Interpretation | What it tests | Rough data moved at 200 bytes per record | At 1 KB per record |
| --- | --- | --- | --- |
| Query execution only | Index and scan speed inside the store | About 1 GB scanned | About 5 GB |
| Records rendered in the console | Query plus API plus browser | About 1 GB through the API | About 5 GB |
| Records exported to a file | Query plus serialization plus disk write | About 1 GB file | About 5 GB file |
| Full delivery to a client machine | All of the above plus the network | About 44 Mbps average | About 222 Mbps average |

Rendering five million rows in a browser is unrealistic for a normal UI, so a sensible test is probably query-and-export. Agree this with OPTCL before testing. Record the exact query, time range, filters, dataset size, concurrent load and the hardware used. Run the test while ingest and detection are running at normal load, not on an idle system.

## 7. Compute and memory sizing method

- Replay traffic at the measured average, the measured peak and 130% of peak (the 30% headroom).

- Record CPU, memory, disk and network use for each tier at each rate.

- Find the first bottleneck. Add capacity to that tier and repeat.

- Set the production size so peak load uses at most 70% of the tested ceiling.

- Add redundancy on top: capacity must remain after losing one node of each clustered tier.

| Tier | Typical first bottleneck to watch |
| --- | --- |
| Packet sensors | Packets per second, NIC queues, disk write |
| Flow collectors | UDP receive buffers, template handling |
| Queue | Disk throughput, network, partition count |
| Workers | CPU, state size for baselines and graph |
| Search index | Memory (heap and cache), disk IOPS, shard count |
| Packet archive | Sequential write throughput, retrieval seek time |

## 8. Input worksheet (to fill in with OPTCL)

| Input | Value | Source | Date |
| --- | --- | --- | --- |
| Average and peak Mbps per observation point |  | Interface counters, 30 days |  |
| Packets per second, average and peak |  | Interface counters or capture |  |
| Packet-size distribution |  | Capture sample |  |
| Flow records per second, average and peak |  | Exporter statistics or sample |  |
| Number of exporters and their flow format |  | Network team |  |
| Share of traffic that is encrypted |  | Capture sample |  |
| Hosts and assets to track |  | Inventory |  |
| Concurrent analyst users |  | SOC |  |
| Concurrent forensic queries expected |  | SOC |  |
| Approved capture filters |  | OPTCL security |  |
| Replication and recovery requirements |  | OPTCL operations |  |
| Rack space, power and cooling available |  | Facilities |  |

## 9. Open items

- Measured traffic profile for every observation point.

- Written definitions of normalized packet data and of record retrieval.

- Confirmation of how the 30% spare capacity is calculated and measured.

- Whether replicas and backups count towards the stated retention periods.

- Hardware platform constraints and approved vendors.